RISK ASSESSMENT

[Location and Event]

| General Location: | [Example A] |  |  | All work to be carried out with accordance with HSE Guidance note HSG150 |
| --- | --- | --- | --- | --- |
| Work Location: | Operation: |  | RA Sheet No: 1 | Date Task Starts: |
| Revision: | Date: | Comments: |  |  |  |

This risk assessment consists of two sections:

- Part A – General Risk Assessments; and

- Part B – Site-Specific Risk Assessments.

The general assessments cover the regular actions and tasks that can occur on all Canal Camps. However, each item needs to be checked, amended if necessary and confirmed as applicable to this activity.

The site-specific assessments cover the actions required for tasks being conducted on this work party. They may be tasks that are conducted on most work parties, but each work party can have different considerations for these tasks; eg: laying bricks may be from a boat or scaffolding and not firm ground.

The layout of the risk assessment allows it to be read piecemeal, i.e. only relevant sections need to be communicated to volunteers engaged in that activity.

Space is available to add hazards, but the reverse of the sheets can be used if there is not enough space on the sheet.

| I have reviewed the general risk assessments in Part A and applied amendments and / or additions. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reviewer | [name] | Role | [role] | Date | [date] |

| List of Sections |  | See Also: | Final Risk Level |  |
| --- | --- | --- | --- | --- |
| Part A | GENERAL | Add reference to toolbox talks, guidance notes, host group documents, etc | L, M H with colour coding |  |
| 1 | Accommodation | Toolbox talk (catering accommodation) | L | M |
| 2 | Unloading the catering kit | Toolbox talk (packing catering cooker) | L |  |
| 3 | Travelling to and from site |  | L | M |
| 4 | Arrival on site | Toolbox talks (slips trips and falls, ground conditions) | L | M |
| 5 | Keeping the public safe | Toolbox talk (the public) | M |  |
| 6 | Setting up the gazebo | Toolbox talk (setting up the gazebo) | L |  |
| 7 | Setting up and using the Burco | Toolbox talk (brew kit and welfare) COSHH assessment 5 | L |  |
| 8 | Unloading the vans and / or trailer | Toolbox talk (manual handling) CRT video | L |  |
| 9 | Working near water | Toolbox talk (working near water) | M |  |
| 10 | Working at height | Toolbox talks (working at height, scaffolding, mobile scaffold tower) Guidance note working at height | M |  |
| 11 | Storage of materials | Toolbox talk (materials storage and handling) | L | M |
| 12 | Use of hand tools |  | L |  |
| 13 | Cleaning bricks and similar materials for re-use | Not applicable |  |  |
| 14 | Use of machinery and power tools (other than plant) | Toolbox talks (Disc-cutters, hand arm vibration, noise) Guidance note | H | M |
| 15 | Use of plant | Not applicable |  |  |
| 16 | Refuelling machinery and plant | Toolbox talk (oil spill kits) COSHH assessments 1, 2 | L |  |
| 17 | Setting up and using the mixer | Not applicable |  |  |
| 18 | Mixing mortar and concrete, including Postcrete. | Not applicable |  |  |
| 19 | Laying blocks | Not applicable |  |  |
| 20 | Moving and laying paving slabs | Not applicable |  |  |
| Part B | SITE-SPECIFIC |  |  |  |
| 21 | Use of chainsaws | Industry rules and guidelines. | H | M |
| 22 | Use of brush cutters |  | H | M |

Likelihood (L):					Severity (S):  							Risk Value  = LxS

Risk Level  High / Medium / Low

| AC | 5 | 10 | 15 | 20 | 25 |
| --- | --- | --- | --- | --- | --- |
| VL | 4 | 8 | 12 | 16 | 20 |
| L | 3 | 6 | 9 | 12 | 15 |
| UL | 2 | 4 | 6 | 8 | 10 |
| EU | 1 | 2 | 3 | 4 | 5 |
|  | NM | Min | >3D | Maj | F |

1	EU   	Extremely Unlikely       		1	NM	Near Miss

2	U     	Unlikely				2	Min	Minor Injury, no time loss

3	L     	Likely				3	>3d    	Time off work more than 3 days

4	VL   	Very Likely			4	Maj      	Major Injury

5	AC   	Almost Certain			5	F      	Fatal

Part A

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | RV | RL |  | L | S | RV | RL |  |
| 1.1 | Accommodation | Kitchen | Cook, volunteers. Cuts, burns and scalds | 2 | 2 | 4 | M | Restrict access to kitchen, cook plus nominated helper. Use oven gloves / mitts when handling hot utensils. Do not place sharp knives in washing up bowls. | 1 | 2 | 2 | L |  |
| 1.2 |  | Spillages | Cook, volunteers. Slips, trips and falls. | 2 | 2 | 4 | M | Keep floors dry, Pick up spilled food. Keep kitchen tidy. | 1 | 2 | 2 | L |  |
| 1.3 |  | Food | Volunteers. illness | 2 | 2 | 4 | M | Keep a check on dates on food packages. Camp-designated cook to have Food Hygiene Level 2 certification. Cook to be aware of allergies and not to mix food / utensils | 1 | 2 | 2 | L |  |
| 1.4 |  | Showers and Toilets | Volunteers. Slips, trips and falls. | 3 | 2 | 6 | M | Keep floors as dry as possible. Keep areas tidy. Use anti-slip mats where possible. Don’t run in changing areas. | 1 | 2 | 2 | L |  |
| 1.5 |  | Boot / wet clothes store | Volunteers. Slips, trips and falls. | 2 | 2 | 4 | M | Keep floors as dry as possible. Keep areas tidy. | 1 | 2 | 2 | L |  |
| 1.6 |  | Heaters | Volunteers. Burns. | 3 | 5 | 15 | H | Don’t cover heaters with the potential to cause a fire. Make volunteers aware of position of heaters. | 1 | 5 | 5 | M |  |
| 2.1 | Unloading and loading the catering kit | Heavy and / or awkward items. | Volunteers. Musculo-skeletal and impact injuries. | 4 | 3 | 12 | H | Use appropriate number of volunteers for each item. Co-ordinate lifting. Only use volunteers capable of lifting the load. Keep carrying distances to a minimum. Avoid twisting with a load. Unload the trailer in order so that unsupported items do not fall. | 1 | 3 | 3 | L | Monitor lifting techniques. Monitor items to ensure they are not left in an unstable position. |
| 3.1 | Travelling to and from site | Walking | Volunteers. Injury from uneven ground or basin’s moving machinery. | 2 | 4 | 8 | M | Remain alert Always look in the direction of movement. | 1 | 4 | 4 | M |  |
| 4.1 | Arrival on site | Traffic | Volunteers, visitors, public. Impact injury from moving vehicles. | 4 | 5 | 20 | H | Ensure there is adequate space off road for people. Do not stand or work on the access track. | 1 | 5 | 5 | M |  |
| 4.2 |  | Weather | Volunteers, visitors. Effects of cold, heat, precipitation. | 4 | 1 | 4 | M | Wear appropriate clothing to suit weather conditions. Use sunscreen on exposed skin on hot sunny days. Take hot / cold drinks and food as appropriate. Establish a shelter for people. | 1 | 1 | 1 | L | Monitor weather conditions during the workday |
| 4.3 |  | Uneven ground | Volunteers, visitors, public. Slips, trips and falls. | 4 | 2 | 8 | M | Take extra care with vegetation and puddles that may hide changes in ground conditions. Wear appropriate footwear. Establish pedestrian routes. | 1 | 2 | 2 | L |  |
| 4.4 |  | Animals | Volunteers. Attack, stings and bites. | 3 | 3 | 9 | M | Don’t disturb insect nests. Wear long clothing. | 1 | 3 | 3 | L |  |
| 4.5 |  | Contaminated material | Volunteers. Skin, eye irritation. Puncture wounds from needles, glass. Disease from faeces. | 3 | 3 | 9 | M | Avoid contact with unknown materials. Wear gloves. Avoid contact with face and eyes with dirty hands. Do not handle needles or glass. Wash hands before eating, drinking or smoking. Cover open wounds. | 1 | 3 | 3 | L |  |
| 5.1 | Keeping the public safe | Site activities | Public. Injury from moving plant and site wide hazards | 3 | 5 | 15 | H | Keep site gate locked to prevent public access. Set up warning signs. | 1 | 5 | 5 | M |  |
| 6.1 | Setting up Gazebo | Gazebo | Volunteers. Entrapment injury. | 3 | 1 | 3 | L | Provide instruction on setting up Gazebo. Keep hands and fingers away from items that may trap them. | 1 | 1 | 1 | L |  |
| 6.2 |  | Gazebo | Volunteers. Musculo-skeletal strains and sprains due to manual handling. | 3 | 2 | 6 | M | Use 4 volunteers to set up gazebo. Co-ordinate lifting. Only use volunteers capable of lifting the load. | 1 | 2 | 2 | L |  |

| 7.1 | Setting up and using the Burco | Burco | Volunteers. Musculo-skeletal strains and sprains due to manual handling. | 3 | 2 | 6 | M | Use 2 volunteers to set up Burco. Provide instruction for setting up Burco. Locate Burco on level ground. Fill Burco after setting up. | 1 | 2 | 2 | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.2 |  | Hot Water and steam | Volunteers. Burns and scalds | 3 | 1 | 3 | L | Instruct volunteers in correct use of Burco. Avoid handling Burco during water heating. Take care when filling to avoid splashes of hot water. Avoid removing lid when water is boiling. Take care when emptying the Burco at the end of the day and packing away hot equipment. | 1 | 1 | 1 | L | Monitor water level in Burco. |
| 8.1 | Unloading vans and / or trailer | Tools and equipment | Volunteers. Manual handling injury. | 3 | 2 | 6 | M | Only use volunteers capable of lifting the load. Avoid stretching and twisting when moving equipment. | 1 | 2 | 2 | L |  |
| 8.2 |  | Tools and equipment | Volunteers. Entrapment, impact injury | 3 | 2 | 6 | M | Unload the trailer in correct order. Only remove straps from the item being unloaded to avoid other items accidentally falling. Wear gloves and appropriate footwear. | 1 | 2 | 2 | L |  |
| 9.1 | Work site near water | Water | Volunteers. Drowning. | 3 | 5 | 15 | H | Take care when working close to water edges to avoid overbalancing. Face water edges. No lone working. Ensure throw lines or lifebuoys are available. Wear lifejacket | 1 | 5 | 5 | M | Do not allow volunteers to work in [specified] areas. |
| 9.2 |  | Water | Volunteers. Weil’s Disease | 3 | 5 | 15 | H | Awareness of Weil’s Disease and carry warning card. Avoid contact with standing water. Cover cuts and open wounds. Avoid touching mouth, nose or eyes. Wash hands before eating, drinking or smoking. | 1 | 5 | 5 | M |  |
| 9.3 |  | Plant / water | Not applicable |  |  |  |  |  |  |  |  |  |  |
| 10.1 | Working at height | Ground | Volunteers. Falls from height. | 4 | 5 | 20 | H | Avoid unprotected edges. Take care when working close to unprotected edges to avoid overbalancing. Face unprotected edges. No lone working | 1 | 5 | 5 | M |  |
| 10.2 |  | Falling objects | Volunteers. Impact injuries. | 4 | 4 | 16 | H | Avoid working beneath areas where work is being carried out. Wear head protection when appropriate. | 1 | 4 | 4 | M | Monitor any work being carried out at height and avoid walking beneath it. |
| 11.1 | Storage of materials | Not applicable |  |  |  |  |  |  |  |  |  |  |  |
| 12.1 | Use of hand tools | Hand tools | Volunteers. Cuts, grazes, impact injury. | 3 | 2 | 6 | M | Use the right tool for the job. Provide training in how to use the tools. Make sure the tool is maintained with no loose parts. Wear gloves. Don’t swing tools within impact range of other people. | 1 | 2 | 2 | L |  |
| 12.2 |  | Hand tools | Volunteers. Manual handling injury | 3 | 2 | 6 | M | Use the right tool for the job. Provide training in how to use the tools. Make sure the tools is sharpened if appropriate. Take breaks to avoid fatigue. | 1 | 2 | 2 | L | Monitor volunteers to make sure breaks are taken |
| 12.3 |  | Ground | Volunteers. Slips and falls while using such tools. | 3 | 2 | 6 | M | Care to be taken when working on wet ground or loose soil. Firm stance to be achieved before commencing work. Don’t swing tools within impact range of other people. | 1 | 2 | 2 | L |  |
| 13.1 | Cleaning bricks and similar materials for re-use | Not applicable |  |  |  |  |  |  |  |  |  |  |  |
| 14.1 | Use of machinery and power tools (other than plant) | Power tools | Volunteers. Cuts, grazes, impact injury. | 3 | 2 | 6 | M | Use the right tool for the job. Provide training in how to use the tools. Make sure the tool is sharpened if appropriate. Wear appropriate PPE. | 1 | 2 | 2 | L | Users of disc-cutters must have been formally trained and such training and subsequent experience recorded. |
| 14.2 |  | Power tools | Volunteers. Manual handling injury | 3 | 2 | 6 | M | Use the right tool for the job. Provide training in how to use the tools. Make sure the tools is sharpened if appropriate. Take breaks to avoid fatigue | 1 | 2 | 2 | L | Monitor volunteers to make sure breaks are taken |
| 14.3 |  | Power tools | Volunteers. Musculo-skeletal injury due to vibration | 3 | 4 | 12 | H | Use the right tool for the job. Ensure the tool is properly maintained. Review the vibration data for the tool, record and limit time spent using the tool. Provide training in the use of the tool. | 1 | 4 | 4 | M | Monitor volunteers to make sure they do not exceed the exposure time. (e.g. Allsaw, vibration 7.7m/s², exposure limit 1hr max use per 8 hours) |
| 14.4 |  | Power tools | Volunteers. Hearing damage from noise | 3 | 4 | 12 | H | Use the right tool for the job. Make sure tool is properly maintained Use ear defenders Provide warning signs. | 1 | 4 | 4 | M |  |
| 14.5 |  | Flying debris and dust, when cutting timber and steel. | Volunteers, Impact injury, eye injury, dust | 2 | 2 | 4 | M | Work away from other workers Wear gloves, eye protection and dust mask. Wear long clothing. | 1 | 2 | 2 | L | Monitor debris generation and dust |
| 15.1 | Use of plant | Not applicable. |  |  |  |  |  |  |  |  |  |  |  |
| 16.1 | Refuelling machinery | Fuel and oil | Volunteers. Skin irritation when refuelling | 3 | 2 | 6 | M | Use funnels when refuelling and filling with oil. Wear gloves. | 1 | 2 | 2 | L |  |
| 16.2 |  | Fuel and oil | Environment, pollution | 4 | 3 | 12 | H | Refuel in designated area. Use drip trays when refuelling. Use funnels when refuelling. | 1 | 3 | 3 | L |  |
| 17.1 | Setting up the mixer | Not applicable. |  |  |  |  |  |  |  |  |  |  |  |
| 18.1 | Mixing mortar and concrete – Bagged and pre-mixed concrete. | Not applicable |  |  |  |  |  |  |  |  |  |  |  |
| 19.1 | Laying blocks | Not applicable. |  |  |  |  |  |  |  |  |  |  |  |
| 20.1 | Laying paving slabs | Not applicable. |  |  |  |  |  |  |  |  |  |  |  |

Part B

| 21.1 | Using a chainsaw | Chainsaw | Operator. Musculo-skeletal injury during use. | 3 | 2 | 6 | M | Only professionally trained operators with sufficient experience to use chainsaws. | 1 | 2 | 2 | L | Industry rules and guidelines to be followed. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21.2 |  | Chainsaw | Operator. Serious injury or death. | 5 | 5 | 25 | H | Only professionally trained operators with sufficient experience to use chainsaws. | 2 | 5 | 10 | M | Industry rules and guidelines to be followed. |
| 21.3 |  | Chainsaw | Bystanders. Serious injury or death. | 5 | 5 | 25 | H | Only professionally trained operators with sufficient experience to use chainsaws. Non-users to stand well away from the saw and operator. | 1 | 3 | 3 | L | Monitor the number of members of the public and choose an alternative time if possible. |
| 22.1 | Using a brush cutter | Brushcutter | Operator. Musculo-skeletal injury during loading. | 3 | 3 | 9 | M | Read and understand the supplied instructions. Only trained operators with sufficient experience to use the brush cutter. | 1 | 3 | 3 | L |  |
| 22.2 |  | Brushcutter | Operator. Cuts or worse from the spinning cutting blade. | 3 | 5 | 15 | H | Read and understand the supplied instructions. Only trained operators with sufficient experience to use the brush cutter. Operator(s) to stand in the correct position. | 2 | 5 | 10 | M |  |
| 22.3 |  | Brushcutter | Bystanders, Serious injury or death. | 5 | 5 | 25 | H | Read and understand the supplied instructions. Only trained operators with sufficient experience to use brush cutter. Non-users to stand well away from the cutter and operator. | 1 | 3 | 3 | L | Monitor the number of members of the public and choose an alternative time if possible. |

| CAN THE JOB PROCEED? (Tick one box) |  |  |  | Safe using existing control measures |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Safe using existing + additional control measures |  |
|  |  |  |  | Not safe to proceed |  |
| ARE THERE ANY LONG-TERM CONTROLS? |  |  |  |  |  |
| ASSESSED BY: | Name (Printed) |  | Name (signed) |  | Date: |
| AUTHORISED BY: | Name (Printed) |  | Name (Signed) |  | Date: |