RISK ASSESSMENT

[Location and Event]

| General Location: | [Example B] |  |  | All work to be carried out with accordance with HSE Guidance note HSG150 |
| --- | --- | --- | --- | --- |
| Work Location: | Operation: |  | RA Sheet No: 1 | Date Task Starts: |
| Revision: | Date: | Comments: |  |  |  |

This risk assessment consists of two sections:

- Part A – General Risk Assessments; and

- Part B – Site-Specific Risk Assessments.

The general assessments cover the regular actions and tasks that can occur on all Canal Camps. However, each item needs to be checked, amended if necessary and confirmed as applicable to this activity.

The site-specific assessments cover the actions required for tasks being conducted on this work party. They may be tasks that are conducted on most work parties, but each work party can have different considerations for these tasks; eg: laying bricks may be from a boat or scaffolding and not firm ground.

The layout of the risk assessment allows it to be read piecemeal, i.e. only relevant sections need to be communicated to volunteers engaged in that activity.

Space is available to add hazards, but the reverse of the sheets can be used if there is not enough space on the sheet.

| I have reviewed the general risk assessments in Part A and applied amendments and / or additions. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reviewer | [name] | Role | [role] | Date | [date] |

| List of Sections |  | See Also: | Final Risk Level |  |
| --- | --- | --- | --- | --- |
| Part A | GENERAL | Add reference to toolbox talks, guidance notes, local society documents | L, M H with colour coding |  |
| 1 | Accommodation | Toolbox talk (catering accommodation) | L |  |
| 2 | Unloading the catering kit | Toolbox talk (packing catering cooker) | L |  |
| 3 | Travelling to and from site |  | L | M |
| 4 | Arrival on site | Toolbox talks (slips trips and falls, ground conditions) | L | M |
| 5 | Keeping the public safe | Toolbox talk (the public) | M |  |
| 6 | Setting up the gazebo | Toolbox talk (setting up the gazebo) | L |  |
| 7 | Setting up and using the Burco | Toolbox talk (brew kit and welfare) COSHH assessment 5 | L |  |
| 8 | Unloading the trailer | Toolbox talk (manual handling) Guidance note; Manual Handling | L |  |
| 9 | Working near water | Toolbox talk (working near water) CRT Video Guidance Note; Working near water | M |  |
| 10 | Working at height | Toolbox talks (working at height, scaffolding) Guidance note; Work at height | M |  |
| 11 | Storage of materials | Toolbox talk (materials storage and handling) | L | M |
| 12 | Use of hand tools |  | L |  |
| 13 | Cleaning bricks and similar materials for re-use | Not used |  |  |
| 14 | Use of machinery and power tools (other than plant) | Toolbox talks (brick saws, brush cutters, hand arm vibration, noise) Guidance note; Hand arm vibration | L | M |
| 15 | Use of plant | Toolbox talks (plant, banksman, banksman signals) Guidance note; Signals | M |  |
| 16 | Refueling machinery and plant | Toolbox talk (oil spill kits) COSHH assessments 1, 2 | L |  |
| 17 | Setting up and using the mixer | Toolbox talks (mixing with a cement mixer, using a diesel mixer, using an electric mixer) | L |  |
| 18 | Mixing mortar (including lime) and concrete | COSHH assessments 3, 4, 6, 7, 8, 10 | L | M |
| 19 | Laying bricks and blocks | COSHH assessments 12, 13 | L | M |
| 20 | Moving and laying coping stones |  | L |  |
| Part B | SITE-SPECIFIC |  |  |  |
| 21 | Assembling concrete reinforcement and waterbar | Reinforcement Diagrams, COSHH assessment 15, waterbar COSHH assessment | L | M |
| 22 | Assembling and disassembling formwork | Frami Product User Guide; Mabey Design and Temporary Works Design Report, COSHH assessment 16 | L | M |
| 23 | Pouring concrete slab and walls | COSHH assessment 14 | L | M |
| 24 | Horses |  | L |  |

| AC | 5 | 10 | 15 | 20 | 25 |
| --- | --- | --- | --- | --- | --- |
| VL | 4 | 8 | 12 | 16 | 20 |
| L | 3 | 6 | 9 | 12 | 15 |
| UL | 2 | 4 | 6 | 8 | 10 |
| EU | 1 | 2 | 3 | 4 | 5 |
|  | NM | Min | >3D | Maj | F |

Likelihood (L):	Severity (S):	Risk Value  = LxS

| 1 | EU | Extremely Unlikely | 1 | NM | Near Miss | Risk Level High / Medium / Low |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | U | Unlikely | 2 | Min | Minor injury, no time lost |  |  |
| 3 | L | Likely | 3 | >3d | Time off work more than 3 days |  |  |
| 4 | VL | Very Likely | 4 | Maj | Major Injury |  |  |
| 5 | AC | Almost Certain | 5 | F | Fatal |  |  |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
| 1.1 | Accommodation | Kitchen | Cook, volunteers. Cuts, burns and scalds | 2 | 2 | 4 | M | Restrict access to cooking area, cook plus nominated helper. Use oven gloves / mitts when handling hot utensils. Do not place sharp knives in washing up bowls. | 1 | 2 | 2 | L |  |
| 1.2 |  | Spillages | Cook, volunteers. Slips, trips and falls. | 2 | 2 | 4 | M | Keep floors dry, Pick up spilled food. Keep kitchen area tidy. | 1 | 2 | 2 | L |  |
| 1.3 |  | Food | Volunteers. illness | 2 | 2 | 4 | M | Keep checking dates on food packages. Camp-designated cook to have Food Hygiene Level 2 certification. Cook to be aware of allergies and not to mix food / utensils | 1 | 2 | 2 | L |  |
| 1.4 |  | Showers and Toilets | Volunteers. Slips, trips and falls. | 3 | 2 | 6 | M | Keep floors as dry as possible. Keep areas tidy. Use anti-slip mats where possible. Don’t run in changing areas. | 1 | 2 | 2 | L |  |
| 1.5 |  | Boot / wet clothes store | Volunteers. Slips, trips and falls. | 2 | 2 | 4 | M | Keep floors as dry as possible. Keep areas tidy. | 1 | 2 | 2 | L |  |
| 1.6 |  | Heaters | Not used |  |  |  |  |  |  |  |  |  |  |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
| 2.1 | Unloading and loading the catering kit | Heavy and / or awkward items. | Volunteers. Musculo-skeletal and impact injuries. | 4 | 3 | 12 | H | Use appropriate number of volunteers for each item. Co-ordinate lifting. Only use volunteers capable of lifting the load. Keep carrying distances to a minimum. Avoid twisting with a load. Unload the trailer in the correct order so that unsupported items do not fall. | 1 | 3 | 3 | L | Monitor lifting techniques. Monitor items to ensure they are not left in an unstable position. |
| 3.1 | Travelling to and from site | Motor vehicles | Volunteers. Breakdown, collision with other vehicles | 2 | 3 | 6 | M | Properly maintained vehicle. Carry out vehicle checks before driving. Take care when joining the main road. | 1 | 3 | 3 | L |  |
| 3.2 |  | Weather | Volunteers. Vehicle collision, loss of control | 3 | 5 | 15 | H | Check weather conditions before travelling. Clear screens to ensure good visibility. Check traffic reports on the radio. Ensure lights are in full working order in case of poor visibility | 1 | 5 | 5 | M | Monitor weather conditions, road conditions and adjust driving to suit. Monitor traffic reports. |
| 3.3 |  | Poor light | Volunteers. Vehicle collision, loss of control. | 2 | 5 | 10 | H | Check vehicle before driving; make sure lights are working and properly adjusted. Carry spare bulbs. Adjust driving to suit light conditions. Plan the journey to travel in daylight. | 1 | 5 | 5 | M | Monitor light conditions. |
| 3.4 |  | Uneven ground poor ground conditions | Volunteers. Vehicle collision, loss of control | 3 | 5 | 15 | H | Check with locals about unmade road conditions. Avoid unmade roads after poor weather conditions. Avoid unmade roads if you are not sure about ground conditions. | 1 | 5 | 5 | M | Monitor weather conditions while on site and judge whether the situation is worsening. |
| 3.5 |  | Volunteers | Volunteers Driver. Distraction while driving leading to loss of control. | 3 | 5 | 15 | H | Ensure volunteers use seat belts and don’t move about vehicle during travel. Ensure volunteers refrain from attracting driver’s attention. Ensure volunteers do not | 1 | 5 | 5 | M |  |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
|  |  |  |  |  |  |  |  | participate in rowdy behaviour. Ensure any singing is in tune and the song is the driver’s genre  |  |  |  |  |  |
| 4.1 | Arrival on site | Traffic | Volunteers, visitors, public. Impact injury from moving vehicles. | 4 | 5 | 20 | H | Ensure there is adequate space off road for people. Do not stand or work on the access route. | 1 | 5 | 5 | M |  |
| 4.2 |  | Weather | Volunteers, visitors. Effects of cold, heat, precipitation. | 4 | 1 | 4 | M | Wear appropriate clothing to suit weather conditions. Use sunscreen on exposed skin on hot sunny days. Take hot / cold drinks and food as appropriate. Establish shelter for people. | 1 | 1 | 1 | L | Monitor weather conditions during the workday. |
| 4.3 |  | Uneven ground | Volunteers, visitors, public. Slips, trips and falls. | 4 | 2 | 8 | M | Take extra care with vegetation and puddles that may hide changes in ground conditions. Wear appropriate footwear. Establish pedestrian routes. | 1 | 2 | 2 | L |  |
| 4.4 |  | Animals | Volunteers. Attack, stings and bites. | 3 | 3 | 9 | M | Don’t disturb insect nests. Wear long clothing. Advise use of insect repellent and non-drowsy sting/bite treatments. | 1 | 3 | 3 | L |  |
| 4.5 |  | Contaminated material | Volunteers. Skin, eye irritation. Puncture wounds from needles, glass. Disease from faeces. | 3 | 3 | 9 | M | Avoid contact with unknown materials. Wear gloves. Avoid contact with face and eyes with dirty hands. Do not handle needles or glass. Wash hands before eating, drinking or smoking. Cover open wounds. | 1 | 3 | 3 | L |  |
| 4.6 |  | Lymes Disease | Volunteers. Infection from ticks in grass areas | 2 | 3 | 6 | M | Avoid long grass where possible. Wear long clothing | 1 | 3 | 3 | L |  |
| 5.1 | Keeping the public safe | Site activities | Public. Grazing animals. | 3 | 5 | 15 | H | Erect barriers to keep the public/animals out of site area. Set up warning signs. | 1 | 5 | 5 | M |  |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
|  |  |  | Injury from moving plant and site wide hazards |  |  |  |  | All plant parked up and isolated out of working hours. Banksman in attendance during plant movement. Cover/fence excavations out of working hours. |  |  |  |  |  |
| 6.1 | Set up Gazebo | Gazebo | Volunteers. Entrapment injury. | 3 | 1 | 3 | L | Provide instruction on setting up Gazebo. Keep hands and fingers away from items that may trap them. | 1 | 1 | 1 | L |  |
| 6.2 |  | Gazebo | Volunteers. Musculo-skeletal strains and sprains due to manual handling. | 3 | 2 | 6 | M | Use 4 volunteers to set up gazebo. Co-ordinate lifting. Only use volunteers capable of lifting the load. | 1 | 2 | 2 | L |  |
| 7.1 | Set up and using the Burco | Burco | Volunteers. Musculo-skeletal strains and sprains due to manual handling. | 3 | 2 | 6 | M | Use 2 volunteers to set up Burco. Provide instruction for setting up Burco. Locate Burco on level ground. Fill Burco after setting up. | 1 | 2 | 2 | L |  |
| 7.2 |  | Hot Water and steam | Volunteers. Burns and scalds | 3 | 1 | 3 | L | Instruct volunteers in correct use of Burco. Avoid handling Burco during water heating. Take care when filling to avoid splashes of hot water. Avoid removing lid when water is boiling. Take care when emptying the Burco at the end of the day and packing away hot equipment. | 1 | 1 | 1 | L | Monitor water level in Burco. |
| 8.1 | Unloading and loading the trailer/van | Tools and equipment | Volunteers. Manual handling injury. | 3 | 2 | 6 | M | Only use volunteers capable of lifting the load. Avoid stretching and twisting when moving equipment. | 1 | 2 | 2 | L |  |
| 8.2 |  | Tools and equipment | Volunteers. Entrapment, impact injury | 3 | 2 | 6 | M | Unload the trailer/van in order. Only remove straps from the item being unloaded to avoid other | 1 | 2 | 2 | L |  |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
|  |  |  |  |  |  |  |  | items accidentally falling. Wear gloves and appropriate footwear. |  |  |  |  |  |
| 9.1 | Work site near water | Water | Volunteers. Drowning. | 3 | 5 | 15 | H | Take care when working close to water edges to avoid overbalancing. Face water edges. No lone working. Ensure there is a safe place to exit the watercourse if anybody falls in. | 1 | 5 | 5 | M | Monitor water conditions if the water level is likely to change. |
| 9.2 |  | Water | Volunteers. Weil’s Disease | 3 | 5 | 15 | H | Awareness of Weil’s Disease and carry warning card. Avoid contact with standing water. Cover cuts and open wounds. Avoid touching mouth, nose or eyes. Wash hands before eating, drinking or smoking. | 1 | 5 | 5 | M |  |
| 9.3 |  | Plant / water | Volunteers. Plant overturning resulting in immersion in water. | 3 | 5 | 15 | H | Banksman to be used when travelling near water. Keep plant a suitable distance from water’s edge. Ensure plant route is stable without ruts. Take extra care or stop working in wet conditions. | 1 | 5 | 5 | M | Monitor condition of traffic route |
| 10.1 | Working at height | Ground | Volunteers. Falls from height. | 4 | 5 | 20 | H | Avoid unprotected edges. Take care when working close to unprotected edges to avoid overbalancing. Face unprotected edges. No lone working Highlight unprotected edges to raise awareness. | 1 | 5 | 5 | M | Daily check that scaffolding and edge protection is intact/correct. |
| 10.2 |  | Falling objects | Volunteers. Impact injuries. | 4 | 4 | 16 | H | Avoid working beneath areas where work is being carried out. Wear head protection when appropriate. | 1 | 4 | 4 | M | Monitor any work being carried out at height and avoid walking beneath it. |
| 10.3 |  | Scaffolding | Volunteers. Falls from | 4 | 5 | 20 | H | Train volunteers on the safe assembly methods. Wear head protection. | 1 | 5 | 5 | M | Daily check that scaffolding and edge protection is intact/correct. |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
|  |  |  | height and impact injuries. |  |  |  |  | Check scaffolding is assembled correctly and completely, and properly signed off, as required.. Take extra care using scaffolding during assembly/disassembly. |  |  |  |  |  |
| 11.1 | Storage of materials | Materials | Volunteers. Manual handling injury. | 3 | 2 | 6 | M | Use mechanical means to unload where possible. Break the load into smaller parts where possible Only use volunteers capable of lifting the load. Take breaks to avoid fatigue. | 1 | 2 | 2 | L |  |
| 11.2 |  | Materials | Volunteers, Entrapment, impact injury from falling stockpiles. | 3 | 3 | 9 | M | Store materials on level ground. Place materials on pallets, timber battens etc to avoid entrapment with the ground. Place loose materials in suitable bunkers or storage bins. | 1 | 3 | 3 | L |  |
| 11.3 |  | Ground | Volunteers. Falls from climbing onto stored materials | 3 | 4 | 12 | H | Do not climb on stacked materials. Use plant to lift materials to ground level. Unload delivery vehicles from ground level. | 1 | 4 | 4 | M |  |
| 12.1 | Use of hand tools | Hand tools | Volunteers. Cuts, grazes, impact injury. | 3 | 2 | 6 | M | Use the right tool for the job. Provide training in how to use the tools. Make sure the tool is maintained with no loose parts Wear gloves. | 1 | 2 | 2 | L |  |
| 12.2 |  | Hand tools | Volunteers. Manual handling injury | 3 | 2 | 6 | M | Use the right tool for the job. Provide training in how to use the tools. Make sure the tools are sharpened if appropriate. Take breaks to avoid fatigue | 1 | 2 | 2 | L | Monitor volunteers to make sure breaks are taken |
| 13.1 | Cleaning bricks and similar materials for re-use | Not used |  |  |  |  |  |  |  |  |  |  |  |
| 14.1 | Use of machinery and power tools (other than plant) | Power tools | Volunteers. Cuts, grazes, impact injury. | 3 | 2 | 6 | M | Use the right tool for the job. Provide training in how to use the tools. Make sure the tool is sharpened if | 1 | 2 | 2 | L |  |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
|  |  |  |  |  |  |  |  | appropriate. Wear gloves. |  |  |  |  |  |
| 14.2 |  | Power tools | Volunteers. Manual handling injury | 3 | 2 | 6 | M | Use the right tool for the job. Provide training in how to use the tools. Make sure the tools are sharpened if appropriate. Take breaks to avoid fatigue | 1 | 2 | 2 | L | Monitor volunteers to make sure breaks are taken |
| 14.3 |  | Power tools | Volunteers. Musculo-skeletal injury due to vibration | 3 | 4 | 12 | H | Use the right tool for the job. Ensure the tool is properly maintained. Record and review the vibration data for the tool and limit time spent using the tool. Provide training in the use of the tool. | 1 | 4 | 4 | M | Monitor volunteers to make sure they do not exceed the exposure time. Brick saw 3.6m/s2, exposure limit 8 hours. Vibrating poker (0.5m from head) 7.8 m/s2, exposure limit 61 minutes. |
| 14.4 |  | Power tools | Volunteers. Hearing damage from noise | 3 | 4 | 12 | H | Use the right tool for the job. Make sure tool is properly maintained Use ear defenders Provide warning signs. | 1 | 4 | 4 | M |  |
| 14.5 |  | Flying debris and dust, when concrete cutting. | Volunteers, Impact injury, eye injury, dust | 2 | 2 | 4 | M | Work away from other workers Use dust suppression, such as water. Wear gloves, eye protection and dust mask. Wear long clothing. | 1 | 2 | 2 | L | Monitor debris generation and dust |
| 15.1 | Use of plant | Plant | Volunteers. Vehicle impact injury | 3 | 5 | 15 | H | Only use authorised staff for plant operation Use banksman to stop plant or pedestrians Segregate plant and pedestrian routes. Make volunteers aware of plant working Wear Hi-Viz clothing Set up warning signs | 1 | 5 | 5 | M |  |
| 15.2 |  | Reversing plant | Volunteers. Impact with people and objects, | 3 | 4 | 12 | H | Use authorised plant operators Travel at a speed that is suitable for the conditions. Look in the direction of travel | 1 | 4 | 4 | M |  |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
|  |  |  | possible overturning of plant |  |  |  |  | Use plant that has a reversing beacon or bleeper Wear Hi-Viz clothing |  |  |  |  |  |
| 15.3 |  | Uneven ground | Volunteers. Injury due to plant overturning | 3 | 5 | 15 | H | Use authorised plant operators Travel at a speed that is suitable for the conditions. Look in the direction of travel | 1 | 5 | 5 | M |  |
| 15.4 |  | Plant | Volunteers. Hearing damage from noise | 3 | 4 | 12 | H | Ensure plant is properly maintained. Ensure engine covers are in place. Use banksman to signal operator. | 1 | 4 | 4 | M |  |
| 16.1 | Refuelling machinery and plant | Fuel and oil | Volunteers. Skin irritation when refuelling | 3 | 2 | 6 | M | Use funnels when refuelling and filling with oil. Wear gloves. | 1 | 2 | 2 | L |  |
| 16.2 |  | Fuel and oil | Environment, pollution | 4 | 3 | 12 | H | Refuel in designated area. Fully bund fuel store. Use drip trays for static plant Use funnels when refuelling. Spill kit to be readily available. | 1 | 3 | 3 | L |  |
| 17.1 | Setting up the mixer | Mixer | Volunteers. Manual handling injury. | 3 | 2 | 6 | M | Only use volunteers capable of lifting the load. Avoid stretching and twisting when positioning the mixer. Place the mixer close to the materials. | 1 | 2 | 2 | L |  |
| 17.2 |  | Mixer | Volunteers. Entrapment, impact injury | 3 | 2 | 6 | M | Set the mixer up on a level surface. Keep hands and fingers away from where they can be trapped. | 1 | 2 | 2 | L |  |
| 18.1 | Mixing mortar and concrete | Cement mortar. | Volunteers. Skin and eye irritation | 4 | 3 | 12 | H | Train volunteers how to mix materials. Provide COSHH assessment Wear eye protection, long sleeves and gloves. Do not stand in front of mixer drum when splashes are likely. Avoid contact with cement / lime | 1 | 3 | 3 | L |  |
| 18.2 |  | Cement and sand dust | Volunteers, public. Respiratory | 4 | 4 | 16 | H | Site mixer in designated area away from other activities and public. Provide screens in windy | 1 | 4 | 4 | M | Monitor wind conditions |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
|  |  |  | problems |  |  |  |  | conditions. Place materials in mixer drum in a controlled manner. Wear dust masks Provide training in use of dust masks |  |  |  |  |  |
| 18.3 |  | Wash water | Environment. Pollution | 4 | 2 | 8 | M | Collect wash water from mixer. Allow solids to settle before disposing of water | 1 | 2 | 2 | L |  |
| 18.4 |  | Cement / sand. Emptying full mixer | Volunteers. Manual handling injury | 3 | 3 | 9 | M | Place materials close to mixer to reduce travel distance. Avoid excessive twisting and lifting. Take regular breaks to avoid fatigue. Training in correct use of mixer and control during emptying. | 1 | 3 | 3 | L | Monitor volunteers for signs of fatigue. |
| 18.5 |  | Mixer / spade | Volunteers. Impact and entrapment from spade catching inside the mixer drum | 3 | 2 | 6 | M | Training in correct use of mixer. Do not allow the spade to enter the mixer drum during operation. | 1 | 2 | 2 | L |  |
| 19.1 | Laying bricks and blocks | Bricks / blocks | Volunteers. Manual handling injury | 3 | 3 | 9 | M | Provide manual handling training. Avoid twisting and bending excessively when laying bricks / blocks. Limit time spent brick or block laying. Take breaks to avoid fatigue. | 1 | 3 | 3 | L | Monitor volunteers for signs of fatigue. |
| 19.2 |  | Cement mortar. | Volunteers. Skin and eye irritation | 4 | 3 | 12 | H | Train volunteers how to use materials. Provide COSHH assessment Wear eye protection, long sleeves and gloves. Avoid skin contact with cement mortar. | 1 | 3 | 3 | L |  |
| 19.3 |  | Dust when cutting bricks or blocks | Volunteers, public. Respiratory problems | 4 | 4 | 16 | H | Cut bricks and blocks in designated area away from public areas. Use water suppression where possible. | 1 | 4 | 4 | M | Monitor weather conditions |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
|  |  |  |  |  |  |  |  | Provide screens in windy conditions. Wear dust masks Provide training in use of dust masks |  |  |  |  |  |
| 20.1 | Laying coping stones | Bricks / blocks | Volunteers. Manual handling injury | 3 | 3 | 9 | M | Use mechanical aids for heavy stones. Provide manual handling training. Avoid twisting and bending excessively when laying coping stones. Limit time spent stone laying. Take breaks to avoid fatigue. | 1 | 3 | 3 | L | Monitor volunteers for signs of fatigue. |
| 20.2 |  | Cement / lime mortar. | Volunteers. Skin and eye irritation | 4 | 3 | 12 | H | Train volunteers how to use materials. Provide COSHH assessment Wear eye protection, long sleeves and gloves. Avoid skin contact with cement / lime mortar. | 1 | 3 | 3 | L |  |
| 20.3 |  | Dust when cutting stones | Volunteers, public. Respiratory problems | 4 | 4 | 16 | H | Cut stones in designated area away from public areas. Provide screens in windy conditions. Wear dust masks Provide training in use of dust masks | 1 | 4 | 4 | M | Monitor weather conditions |
| 21.1 | Assembling concrete reinforcement and waterbar | Rebar rods and mesh | Volunteers. Injury due to impact or sharp ends edges/during manual handling | 3 | 4 | 12 | H | Train volunteers on how to handle materials and hazard awareness. Wear eye protection and gloves. Avoid skin contact when carrying. Plan route through site and ensure it remains clear of obstructions and other volunteers are aware. Provide COSHH assessment. | 1 | 4 | 4 | M |  |
| 21.2 |  | Rebar rods, mesh, ties and flying offcuts | Volunteers. Injury from sharp ends/edges protruding from construction | 4 | 4 | 16 | H | Train volunteers on how to handle materials and hazard awareness. Wear eye protection. Wear rigger gloves when tying and handling rods. Ensure protruding rods are protected by mushroom caps and that these are taped in place. | 1 | 4 | 4 | M | Regularly check caps are in place. Remove caps only as and when required, not all in one go. |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
| 21.3 |  | Waterbar materials | Volunteers. Skin and eye irritation | 3 | 2 | 6 | M | Train volunteers how to use materials. Wear nitrile gloves when applying. Provide COSHH assessment. | 1 | 2 | 2 | L |  |
| 22.1 | Assembling and disassembling formwork | Frami panels, clamps, clips | Volunteers. Injury due to impact or entrapment during manual handling | 3 | 4 | 12 | H | Train volunteers on safe assembly methods in Product User Manual. Keep hands and fingers away from edges/gaps/sockets. Wear gloves where appropriate. | 1 | 4 | 4 | M |  |
| 22.2 |  | Frami panels release agent / mould oil | Volunteers. Skin and eye irritation | 4 | 2 | 8 | M | Train volunteers how to use materials. Wear nitrile gloves when applying oil. Wear gloves when handling panels. Provide COSHH assessment. | 1 | 2 | 2 | L |  |
| 23.1 | Pouring concrete slab and walls | Concrete pump boom and pipes | Volunteers. Injury from pipe whiplash | 3 | 4 | 12 | H | Minimise number of volunteers in proximity to work. Monitor and supervise pumping to identify risk events and instruct on how any blockage may be safely cleared. | 1 | 4 | 4 | M |  |
| 23.2 |  | Concrete being pumped/vibrate d/floated | Volunteers. Skin and eye irritation | 4 | 3 | 12 | H | Train volunteers how to use materials. Minimise number of volunteers in proximity to work. Provide COSHH assessment. Wear eye protection, paper suits and gloves. Wear safety wellington boots if standing in concrete. Avoid skin contact with concrete. | 1 | 3 | 3 | L |  |
| 23.3 |  | Catastrophic failure of formwork | Volunteers. Injury due to impact or entrapment. | 2 | 5 | 10 | M | Minimise number of volunteers in proximity to work. Review design fits with product rules on configuration, spacing, etc. Pre-pour inspection to check all components are in place and all clamp, clips and ties are secure. | 1 | 4 | 4 | M | Volunteers to monitor formwork for leakage during pour. Stop pour and remediate if leakage observed. |
| 24.1 | Horses and Livestock | Horses and livestock | Volunteers. Injury due to impact. Animals. | 4 | 3 | 12 | H | Ensure worksite is securely fenced with Heras panels, and if access is required through the fenceline panels, they should only be opened for the required time. Close access gates to prevent horses and livestock from straying out of field. | 1 | 3 | 3 | L |  |

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | R V | RL |  | L | S | R V | R L |  |
| 24.2 |  | Horses and livestock | Damage to installed works | 4 | 2 | 8 | M | Ensure worksite is securely fenced with Heras panels and if access is required through fenceline panels are only opened for the required time. Minimise work activities that require access into the field. | 1 | 2 | 2 | L |  |

| CAN THE JOB PROCEED? (Tick one box) |  |  |  | Safe using existing control measures |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Safe using existing + additional control measures |  |
|  |  |  |  | Not safe to proceed |  |
| ARE THERE ANY LONG-TERM CONTROLS? |  | Carry out a dynamic Risk Assessment when you arrive on site and adapt the Risk Assessment to suit any changes that may have occurred, particularly weather conditions. |  |  |  |
| ASSESSED BY: | Name (Printed) |  | Name (signed) |  | Date: |
| AUTHORISED BY: | Name (Printed) |  | Name (Signed) |  | Date: |