RISK ASSESSMENT

Example Canal Clean-up

| Waterway: | Site address: |  | All work to be carried out with accordance with HSE Guidance note HSG150 |
| --- | --- | --- | --- |
| Work Location: | Operation: | RA Sheet No: | Date Task Starts: |

Likelihood (L):					Severity (S):  						Risk Value (RV)  = LxS

1	EU   	Extremely Unlikely       		1	FAC  	First Aid Required			Risk Level (RL)  High / Medium / Low

2	U     	Unlikely				2	LTI     	Lost time injury

| AC | 5 | 10 | 15 | 20 | 25 |
| --- | --- | --- | --- | --- | --- |
| VL | 4 | 8 | 12 | 16 | 20 |
| L | 3 | 6 | 9 | 12 | 15 |
| UL | 2 | 4 | 6 | 8 | 10 |
| EU | 1 | 2 | 3 | 4 | 5 |
|  | FAC | LTI | >3D | MI | F |

3	L     	Likely				3	>3d    	Time off work more than 3 days

4	VL   	Very Likely			4	MI      	Major Injury

5	AC   	Almost Certain			5	F      	Fatal

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | RV | RL |  | L | S | RV | RL |  |
| 1 | General site considerations | Traffic on nearby roads | Volunteers, visitors, public. Impact injury from moving vehicles. | 4 | 5 | 20 | H | Ensure there is adequate space off road for people. Do not stand or work in road without traffic control. Wear Hi-Viz clothing. Ensure members of the public are not diverted onto a road. | 1 | 5 | 5 | M |  |
| 2 |  | Weather | Volunteers, visitors. Effects of cold, heat, precipitation. | 4 | 1 | 4 | M | Wear appropriate clothing to suit weather conditions. Use sunscreen on exposed skin on hot sunny days. Take hot / cold drinks and food as appropriate. Establish a shelter for people. | 1 | 1 | 1 | L | Monitor weather conditions during the workday |
| 3 |  | Uneven ground | Volunteers, visitors, public. Slips, trips and falls. | 4 | 2 | 8 | M | Take extra care with vegetation and puddles that may hide changes in ground conditions. Wear appropriate footwear. Establish pedestrian routes. Ensure public routes are not disturbed. | 1 | 2 | 2 | L |  |
| 4 |  | Public | Volunteers. Aggressive behaviour | 2 | 4 | 8 | M | Park site vehicles off public routes. Do not congregate on public routes. Do not erect shelters on public routes. When dealing with aggression from members of the public, avoid confrontation. Erect warning signs. | 1 | 4 | 4 | L |  |
| 5 |  | Animals | Volunteers. Attack, stings and bites. | 3 | 3 | 9 | M | Avoid entering areas where livestock may be kept. Don’t disturb insect nests. Proceed with caution where animals are loose. Wear long clothing. Do not approach dogs off lead. | 1 | 3 | 3 | L |  |
| 6 | Working near water | Water | Volunteers. Drowning. | 3 | 5 | 15 | H | Wear life jackets. Avoid walking close to unprotected water edges. Take care when working close to water edges to avoid overbalancing. Face water edges. No lone working. Ensure throw lines and lifebuoys are available. | 1 | 5 | 5 | M | Monitor water conditions if the water level is likely to change. |
| 7 |  | Water | Volunteers. Weil’s Disease | 3 | 5 | 15 | H | Awareness of Weil’s Disease and carry warning card. Avoid contact with standing water. Cover cuts and open wounds. Avoid touching mouth, nose or eyes. Wash hands before eating, drinking or smoking. | 1 | 5 | 5 | M |  |
| 8 | Recovering submerged material | Contaminated material | Volunteers. Skin, eye irritation. | 3 | 3 | 9 | M | Avoid contact with unknown materials. Wear gloves. Avoid contact with face and eyes with dirty hands. Do not handle needles or glass. Wash hands before eating, drinking or smoking. Cover open wounds. | 1 | 3 | 3 | L |  |
| 9 |  | Sharp objects | Volunteers Puncture wounds from needles, glass, sharp objects (Knives etc). | 3 | 3 | 9 | M | Avoid contact with sharp objects Wear gloves | 1 | 3 | 3 | L |  |
| 10 |  | Material | Volunteers. Musculo-skeletal injury from heavy load. | 4 | 3 | 12 | H | If submerged obstacle is difficult to move get assistance. Do not try to move too heavy loads. Draw submerged item to the side and seek assistance. Take regular breaks to avoid fatigue. | 2 | 3 | 6 | M |  |
| 11 |  | Water | Volunteers, public. Slips, trips and falls due to wet areas | 4 | 3 | 12 | H | Take extra care when working in wet areas. Use suitable footwear. Use sand to reduce slippery surface. Avoid spreading water over too much of the towpath No lone working. | 2 | 3 | 6 | M | Monitor work area and move volunteers if ground is becoming too wet and slippery. |
| 12 |  | Material | Volunteers, public. Slips trips and falls. | 3 | 3 | 9 | M | Don’t allow material to stockpile on access routes. Keep site tidy Move material to separate stockpile or container. Don’t try clambering over piles of material removed from canal. | 1 | 3 | 3 | L | Monitor size of stockpiles and make sure material is moved to a separate area. |
| 13 |  | Unexploded ordnance | Volunteers. Blast injuries | 2 | 5 | 10 | H | Avoid impact damage to any unexploded ordnance Isolate any ordnance found, maintain exclusion area and notify authorities | 1 | 5 | 5 | M |  |
| 14 | Use of grappling hook and line | Grappling hook | Volunteers. Impact injury | 3 | 3 | 9 | M | Demonstrate correct way to cast the grappling hook. Do not work too close to others. | 1 | 3 | 3 | L |  |
| 15 | Transferring material to skip or hopper | Material | Volunteers. Musculo-skeletal injury from heavy load. | 4 | 3 | 12 | H | If obstacle is difficult to move get assistance. Do not try to move too heavy loads. Avoid twisting when carrying heavy load Avoid over-reaching Take regular breaks to avoid fatigue | 1 | 3 | 3 | L |  |
| 16 |  | Sharp material | Volunteers. Cuts, abrasions, puncture wounds | 3 | 3 | 9 | M | Avoid handling sharp objects Wear gloves | 1 | 3 | 3 | L |  |
| 17 |  | Contaminated material | Volunteers. Skin, eye irritation. | 3 | 3 | 9 | M | Avoid contact with unknown materials. Wear gloves. Avoid contact with face and eyes with dirty hands. Do not handle needles or glass. Wash hands before eating, drinking or smoking. Cover open wounds. | 1 | 3 | 3 | L |  |

| CAN THE JOB PROCEED? (Tick one box) |  |  |  | Safe using existing control measures |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Safe using existing + additional control measures |  |
|  |  |  |  | Not safe to proceed |  |
| ARE THERE ANY LONG-TERM CONTROLS? |  | Carry out a dynamic Risk Assessment when you arrive on site and adapt the Risk Assessment to suit any changes that may have occurred, particularly weather conditions. |  |  |  |
| ASSESSED BY: | Name: |  | Name (signed) |  | Date: |
| AUTHORISED BY: | Name (Printed) |  | Name (Signed) |  | Date: |

Guidance on Completing a Risk assessment

Part of managing health and safety on site is controlling the risks in the workplace. You need to think about what may cause harm to people and decide whether you are taking reasonable steps to prevent that harm. This is known as a risk assessment and is something that you are required to carry out by law.

A risk assessment is not about creating huge amounts of paperwork, but rather about identifying sensible measures to control the risks in your workplace.

Think about how accidents and ill-health could happen and concentrate on real risks – those that are most likely and which could cause the most harm.

Definitions

 Hazard:  Anything that may cause harm, such as chemicals, electricity, working at height or near water, uneven ground, plant and tools, etc.

 Risk: The chance, high or low, that somebody could be harmed by these and other hazards, together with an indication of how serious the harm could be

Steps to writing a Risk assessment

For any task or activity to be undertaken:

- Identify the hazards. Think about the activities, processes or substances that could injure your volunteers or harm their health.

- Decide who might be harmed and how.  For each hazard you need to be clear who might be harmed, volunteers and others not carrying out the task.

- Evaluate the risks and decide on controls to reduce the risk. How likely is the hazard to cause harm and what would the severity be. What can be done to reduce the risk, you do not need to eliminate the hazard.

- Record your significant findings.  Make a record of the hazards, how people may be harmed and what you have in place to control the risks. Any record should be simple and focused and communicated to everyone involved in the task.

- Review your assessment and update if necessary. If there have been any significant changes or improvements need to be made. Consult your volunteers. Learn from any accidents or near misses.

Using this Example Risk Assessment

The intention of the author should be to try to identify all possible hazards for this activity.

Firstly, the heading boxes need to be completed with the details of where you intend to work and what you plan to do.

Then consider all the hazards that are present on your site and complete a line on the schedule for each specific risk accordingly and then assess that risk. You will probably then need to consider possible control measures to reduce the risk to an acceptable level.  Complete the columns on the right-hand side.