RISK ASSESSMENT

Example Risk Assessment for laying a towpath

| Waterway: | Site address: |  | All work to be carried out with accordance with HSE Guidance note HSG150 |
| --- | --- | --- | --- |
| Work Location: | Operation: | RA Sheet No: | Date Task Starts: |

Likelihood (L):					Severity (S):  						Risk Value (RV)  = LxS

1	EU   	Extremely Unlikely       		1	FAC  	First Aid Required			Risk Level (RL)  High / Medium / Low

2	U     	Unlikely				2	LTI     	Lost time injury

| AC | 5 | 10 | 15 | 20 | 25 |
| --- | --- | --- | --- | --- | --- |
| VL | 4 | 8 | 12 | 16 | 20 |
| L | 3 | 6 | 9 | 12 | 15 |
| UL | 2 | 4 | 6 | 8 | 10 |
| EU | 1 | 2 | 3 | 4 | 5 |
|  | FAC | LTI | >3D | MI | F |

3	L     	Likely				3	>3d    	Time off work more than 3 days

4	VL   	Very Likely			4	MI      	Major Injury

5	AC   	Almost Certain			5	F      	Fatal

| No. | TASK | HAZARD | WHO IS EXPOSED AND HOW | RISK LEVEL |  |  |  | CONTROL MEASURES | NEW RISK LEVEL |  |  |  | FURTHER ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L | S | RV | RL |  | L | S | RV | RL |  |
| 1 | General Site Considerations | Traffic | Volunteers, visitors, public. Impact injury from moving vehicles. | 4 | 5 | 20 | H | Ensure there is adequate space off road for people. Do not stand or work in road without traffic control. Wear Hi-Viz clothing. Ensure members of the public are not diverted onto a road. | 1 | 5 | 5 | M |  |
| 2 |  | Weather | Volunteers, visitors. Effects of cold, heat, precipitation. | 4 | 1 | 4 | M | Wear appropriate clothing to suit weather conditions. Use sunscreen on exposed skin on hot sunny days. Take hot / cold drinks and food as appropriate. Establish a shelter for people. | 1 | 1 | 1 | L | Monitor weather conditions during the work day |
| 3 |  | Uneven ground | Volunteers, visitors, public. Slips, trips and falls. | 4 | 2 | 8 | M | Take extra care in vegetation and puddles that may hide changes in ground conditions. Wear appropriate footwear. Establish pedestrian routes. Ensure public routes are not disturbed. | 1 | 2 | 2 | L |  |
| 4 |  | Public | Volunteers. Aggressive behaviour | 2 | 4 | 8 | M | Park site vehicles off public routes. Do not congregate on public routes. Do not erect shelters on public routes. When dealing with aggression from members of the public, avoid confrontation. | 1 | 4 | 4 | M |  |
| 5 |  | Animals | Volunteers. Attack, stings and bites. | 3 | 3 | 9 | M | Avoid entering areas where livestock may be kept. Don’t disturb insect nests. Proceed with caution where animals are loose. Wear long clothing. Do not approach dogs off lead. | 1 | 3 | 3 | L |  |
| 6 |  | Contaminated material | Visitors, volunteers. Skin, eye irritation. Puncture wounds from needles, glass. Disease from faeces. | 3 | 3 | 9 | M | Avoid contact with unknown materials. Wear gloves. Avoid contact with face and eyes with dirty hands. Do not handle needles or glass. Wash hands before eating, drinking or smoking. Cover open wounds. | 1 | 3 | 3 | L |  |
| 7 | Work site near water | Water | Volunteers. Drowning. | 3 | 5 | 15 | H | Avoid working close to unprotected water edges. Wear life jacket if working close to unprotected water edges. Take care when working close to water edges to avoid overbalancing. Face water edges. No lone working. Ensure throw lines or lifebuoys are available. Wear a lifejacket. | 1 | 5 | 5 | M | Monitor water conditions if the water level is likely to change. |
| 8 |  | Water | Volunteers. Weil’s Disease | 3 | 5 | 15 | H | Awareness of Weil’s Disease and carry warning card. Avoid contact with standing water. Cover cuts and open wounds. Avoid touching mouth, nose or eyes. Wash hands before eating, drinking or smoking. | 1 | 5 | 5 | M |  |
| 9 |  | Water | Volunteers. Drowning from flood or tidal water. | 3 | 5 | 15 | H | Check tides and possible flood conditions prior to starting work. No lone working. | 1 | 5 | 5 | M | Monitor weather and tidal conditions |
| 10 | Setting up material storage | Materials | Volunteers. Manual handling injury. | 3 | 2 | 6 | M | Use mechanical means to unload where possible. Only use volunteers capable of lifting the load. Take breaks to avoid fatigue. | 1 | 2 | 2 | L |  |
| 11 |  | Materials | Volunteers, public. Entrapment, impact injury from falling stockpiles. | 3 | 3 | 9 | M | Store materials on level ground. Place materials on pallets, timber battens etc to avoid entrapment with the ground. Place loose materials in suitable bunkers or storage bins. Site materials away from public areas. | 1 | 3 | 3 | L |  |
| 12 | Excavation for towpath | Uneven or sloping ground | Volunteers. Slips trips and falls. Plant overturning | 3 | 4 | 12 | H | Take extra care with vegetation and puddles that may obscure obstructions. Avoid driving plant across slopes if too steep. Avoid overloading plant or uneven loads. Use excavator to level route for pedestrians and plant Wear appropriate footwear | 1 | 4 | 4 | M | Monitor ground and slope conditions |
| 13 |  | Tree roots across towpath | Volunteers. Slips trips and falls. Plant overturning | 3 | 4 | 12 | H | Identify positions of tree roots across the towpath Avoid driving plant over roots. Remove the roots if possible or build up towpath over roots | 2 | 4 | 8 | M |  |
| 14 |  | Buried or overhead services | Volunteers. Explosion, electrocution, release of high-pressure water | 2 | 5 | 10 | H | Carry out service search and mark possible location on the ground Set up marker posts for overhead services CAT scan before excavation Hand dig where CAT identifies services | 1 | 5 | 5 | M |  |
| 15 |  | Plant | Volunteers. Impact with plant | 3 | 4 | 12 | H | Use authorised plant operators Use banksmen to control plant operations Attract plant operators’ attention when passing plant Do not pass plant until it has been stopped and isolated Wear Hi-Viz clothing | 1 | 4 | 4 | M |  |
| 16 |  | Reversing plant | Volunteers. Impact with people and objects, possible overturning of plant | 3 | 4 | 12 | H | Plant operators must have relevant training and experience for the work to be performed. Travel at a speed that is suitable for the conditions. Look in the direction of travel Use plant that has a reversing beacon or bleeper Wear Hi-Viz clothing | 1 | 4 | 4 | M |  |
| 17 |  | Stockpiles of excavated material | Volunteers. Slips trips and falls | 3 | 3 | 9 | M | Avoid walking over stockpiled material Stockpile material away from access routes | 1 | 3 | 3 | L | Monitor the size and site of stockpiles |
| 18 | Placing edging timber | Use of hand tools | Volunteers. Cuts, grazes, abrasions, impact injury | 3 | 2 | 6 | M | Use the right tool for the job. Provide training in how to use the tools. Wear gloves. | 1 | 2 | 2 | L |  |
| 19 |  | Timber | Volunteers. Cuts, splinters, pinching injury | 3 | 2 | 6 | M | Avoid running bare hands along timber. Don’t put fingers between timbers being fixed together. Wear gloves. | 1 | 2 | 2 | L |  |
| 20 |  | Timber | Volunteers. Impact injury when timber being moved | 2 | 2 | 4 | M | Don’t swing lengths of timber where others are working | 1 | 2 | 2 | L |  |
| 21 | Laying geotextile | Geotextile | Volunteers. Manual handling injury | 3 | 2 | 6 | M | Avoid lifting loads that are too heavy Avoid twisting when moving loads | 1 | 2 | 2 | L |  |
| 22 |  | Cutting tools | Volunteers. Cuts | 3 | 2 | 6 | M | Use the right tool for the job Keep hands away from cutting edges Cut on a cutting surface | 1 | 2 | 2 | L |  |
| 23 | Filling towpath | Filling material | Volunteers. Crush injury from tipped material | 2 | 2 | 4 | M | Keep away from plant tipping material | 1 | 2 | 2 | L |  |
| 24 |  | Uneven ground when material tipped | Volunteers. Slips trips and falls | 3 | 2 | 6 | M | Avoid walking over loose tipped material Wear suitable footwear | 1 | 2 | 2 | L |  |
| 25 |  | Filling material | Volunteers. Manual handling injury during spreading | 3 | 2 | 6 | M | Use mechanical means for spreading where possible Use the right tool for the job Limit time spent spreading material Take breaks to avoid fatigue | 1 | 2 | 2 | L | Monitor volunteers to make sure they take enough breaks |
| 26 |  | Dust | Volunteers, public. Respiratory problems, eye irritation | 3 | 3 | 9 | M | Avoid tipping fill from height If dust is a problem, then spray water on stockpiles Consider setting up screen to protect public access routes | 1 | 3 | 3 | L | Monitor weather conditions and condition of fill material |
| 27 | Compacting fill material | Compacting plant | Volunteers. Vibration injury | 3 | 3 | 9 | M | Use the right plant for the job Train volunteers in the use of the plant Limit and record time or activity Take regular breaks to avoid overexposure | 1 | 3 | 3 | L | Monitor volunteers to make sure they take enough breaks |
| 28 |  | Compacting plant | Volunteers, public. Damage to hearing due to noise | 3 | 3 | 9 | M | Use properly maintained plant Wear ear defenders Provide warning signs and diversion route Use a banksman to stop plant if too many people are in the vicinity during compaction | 1 | 3 | 3 | L | Monitor pedestrian movements and carry out activity at an alternative time. |
| 29 |  | Compacting plant | Volunteers. Crush injury from being run over by plant | 2 | 4 | 8 | M | Only volunteers using plant to be in the area Use plant as instructed Don’t walk in front of compacting plant during operation. Wear H-Viz clothing | 1 | 4 | 4 | M |  |
| 30 | Working at height (next to a bank supporting wall or steeply sloping bank) | Ground | Volunteers. Falls from height. | 4 | 5 | 20 | H | Avoid unprotected edges. Take care when working close to unprotected edges to avoid overbalancing. Face unprotected edges. No lone working | 1 | 5 | 5 | M |  |
| 31 |  | Structural collapse | Volunteers. Impact injury from collapsing structure, falls and entrapment from collapsing excavation or canal bank. | 2 | 5 | 10 | M | Avoid unsafe structures. Avoid unsupported excavations. Keep away from edges. No Lone Working | 1 | 5 | 5 | M | Monitor condition of structures. Monitor weather conditions. Monitor vehicle and plant movements near excavations. |

| CAN THE JOB PROCEED? (Tick one box) |  |  |  | Safe using existing control measures |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Safe using existing + additional control measures |  |
|  |  |  |  | Not safe to proceed |  |
| ARE THERE ANY LONG-TERM CONTROLS? |  | Carry out a dynamic Risk Assessment when you arrive on site and adapt the Risk Assessment to suit any changes that may have occurred, particularly weather conditions. |  |  |  |
| ASSESSED BY: | Name: |  | Name (signed) |  | Date: |
| AUTHORISED BY: | Name (Printed) |  | Name (Signed) |  | Date: |

Guidance on Completing a Risk assessment

Part of managing health and safety on site is controlling the risks in the workplace. You need to think about what may cause harm to people and decide whether you are taking reasonable steps to prevent that harm. This is known as a risk assessment and is something that you are required to carry out by law.

A risk assessment is not about creating huge amounts of paperwork, but rather about identifying sensible measures to control the risks in your workplace.

Think about how accidents and ill-health could happen and concentrate on real risks – those that are most likely and which could cause the most harm.

Definitions

 Hazard:  Anything that may cause harm, such as chemicals, electricity, working at height or near water, uneven ground, plant and tools, etc.

 Risk: The chance, high or low, that somebody could be harmed by these and other hazards, together with an indication of how serious the harm could be

Steps to writing a Risk assessment

For any task or activity to be undertaken:

- Identify the hazards. Think about the activities, processes or substances that could injure your volunteers or harm their health.

- Decide who might be harmed and how.  For each hazard you need to be clear who might be harmed, volunteers and others not carrying out the task.

- Evaluate the risks and decide on controls to reduce the risk. How likely is the hazard to cause harm and what would the severity be. What can be done to reduce the risk, you do not need to eliminate the hazard.

- Record your significant findings.  Make a record of the hazards, how people may be harmed and what you have in place to control the risks. Any record should be simple and focused and communicated to everyone involved in the task.

- Review your assessment and update if necessary. If there have been any significant changes or improvements need to be made. Consult your volunteers. Learn from any accidents or near misses.

Using this Example Risk Assessment

The intention of the author should be to try to identify all possible hazards for this activity.

Firstly, the heading boxes need to be completed with the details of where you intend to work and what you plan to do.

Then consider all the hazards that are present on your site and complete a line on the schedule for each specific risk accordingly and then assess that risk. You will probably then need to consider possible control measures to reduce the risk to an acceptable level.  Complete the columns on the right-hand side.